PARIS MOU将于2005年9-11月实施GMDSS集中大检查

据悉，PARIS MOU将于2005年9月至11月开展GMDSS集中大检查，检查将涉及设备功能测试、验证GMDSS操作员对设备的熟悉情况、及要求操作员进行实操。下表内容，供各船舶在迎检时参考。

特此通报。

                          中国船级社营运入级处    2005年7月12日

	No.
	Question

	1
	无线电证书有效，GMDSS满足船舶航区的要求。

Radio certification valid and GMDSS compliant for the sea areas the ship is operating?

	2
	船舶是否有免除证书？船舶是否符合免除的特殊要求？

If an exemption certificate been issued?
Does the ship comply with special requirements imposed by the exemption?

	3
	船舶配备的设备是否符合无线电证书？

Is the equipment fitted in accordance with radio certification?

	4
	GMDSS操作员是否符合无线电证书要求？

Number of GMDSS operators on board meets certificates’ requirement?

	5
	操作员能否正确使用GMDSS装置的各种设备，并进行测试？

Operators able to use all components of the arrangements including test functions?

	6
	误发遇险警报后，GMDSS操作员是否能解释如何纠正？

GMDSS operator(s) able to explain correct procedures in cancelling a false distress alert?

	7
	无线电装置测试是否满意？

Operation tests of radio installation satisfactory?

	8
	海上安全信息船舶是否能够接受？

MSI messages received by ship?

	9
	卫星应急无线示位标装置状况良好？

EPIRB installation satisfactory?

	10
	雷达应答器装置状况是否良好？

Radar transponder installation satisfactory?

	11
	天线状况是否良好？

Antenna condition satisfactory?

	12
	无线电设备电池状况是否良好？

Radio batteries condition satisfactory?


