中  国  船  级  社

CHINA CLASSIFICATION SOCIETY

ENHANCED SURVEY PROGRAMME

FOR TANKERS

Basic information and particulars

| Ship’s name: |  |  |
| --- | --- | --- |
| Kind of Survey: | No. Special Survey |  |
|  | Intermediate Survey in scope of No. Special Survey |  |
| IMO number: |  |  |
| Flag State: |  |  |
| Port of registry: |  |  |
| Gross tonnage: |  |  |
| Deadweight (metric tonnes): |  |  |
| Length between perpendiculars(m): |  |  |
| Shipbuilder: |  |  |
| Hull No.: |  |  |
| Recognized organization (RO): |  | China Classification Society |
| Class No.: |  |  |
| Class Character and Notations of Hull: |  |  |
| Date of build of the ship: |  |  |
| Owner: |  |  |
| Thickness measurement firm: |  |  |

# CONTENTS

1. Preamble	3

2. Arrangement of tanks and spaces	3

3. List of tanks and spaces with information on their use, extent of coatings and corrosion protection      system	3

4. Conditions for survey	4

5. Provisions and method of access to structures	4

6. List of equipment for survey	6

7. Survey requirements	6

8. Identifications of tanks for tank testing	7

9. Identification of areas and sections for thickness measurements	8

10. Minimum thickness of hull structures	8

11. Thickness measurements firm	8

12. Damage experience related to the ship	9

13. Areas identified with substantial corrosion from previous surveys	10

14. Critical structural areas and suspect areas	10

15. Other relevant comments and information	10

Appendices	11

Appendix 1 - List of Plans	12

Appendix 2 - Survey Planning Questionnaire to Survey Programme	13

Appendix 3.1 - Corrosion And Wastage Allowance of Hull Structure	17

Appendix 3.2 - Minimum Thickness Table of Hull Structure	18

# 1. Preamble

1.1 Scope

1.1.1 The present survey programme covers the minimum extent of overall surveys, close-up surveys, thickness measurements and pressure testing within the cargo area, ballast tanks including fore and aft peak tanks, required by CCS Rules and 2011 ESP Code.

1.1.2 The arrangements and safety aspects of the survey shall be acceptable to the attending surveyor(s).

1.2 Documentation

All documents used in the development of the survey programme shall be available onboard during the survey as required by 5.1.6.5(2)①, Section 1, Chapter 5, PART ONE of CCS Rules.

# 2. Arrangement of tanks and spaces

This section of the survey programme shall provide information (either in the form of plans or text) on the arrangement of cargo holds, tanks and spaces that fall within the scope of the survey.

# 3. List of tanks and spaces with information on their use, extent of coatings and corrosion prevention system

This section of the survey programme shall indicate any changes relating to (and shall update) the information on the use of the tanks of the ship, the extent of coatings and the corrosion protective system provided in the survey planning questionnaire.

| Tank/ Hold No. | Corrosion Protection ① |  |  |  |  | Coating Extent ② |  |  |  | Coating Condition ③ |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | HC | SH | SC | A | NP | U | M | L | C | G | F | P | RC |

Abbreviation:

① HC-N: Hard coating not subject to PSPC and PSPC (Void Space)

HC-B: Hard coating applied in dedicated seawater ballast tanks (PSPC)

HC-C: Hard coating applied in cargo oil tank spaces (PSPC)

HC-V: Hard coating applied in void spaces (PSPC)

SH = Semi Hard Coating    SC = Soft coating    A = Anodes    NP = No protection

② U=Upper part    M=Middle part    L=Lower part    C=Complete

③ G=Good    F=Fair    P=Poor    RC=Recoated (during the last 3 years)

Notes:

(1) To be filled with applicable type of hard coating in column mark with “HC”;

(2) To be filled with “X” in all columns except for “HC” as applicable;

(3) For the definition of the coating condition, refer to CCS Rule.

# 4. Conditions for survey

4.1 The owner shall provide the necessary facilities for a safe execution of the survey.

4.1.1 In order to enable the attending surveyors to carry out the survey, provisions for proper and safe access shall be agreed between the owner and China Classification Society, based on IMO Resolution A.1050(27) -- Revised recommendations for entering enclosed spaces aboard ships.

4.1.2 In cases where the provisions of safety and required access are judged by the attending surveyor(s) not to be adequate, the survey of the spaces involved shall not proceed.

4.2 Tanks and spaces are shall safe for access. Tanks and spaces shall be gas free and properly ventilated. Prior to entering a tank, void or enclosed space, it shall be verified that the atmosphere in that space is free from hazardous gas and contains sufficient oxygen.

4.3 In preparation for survey and thickness measurements and to allow for a thorough examination, all spaces shall be cleaned including removal from surfaces of all loose accumulated corrosion scale. Spaces shall be sufficiently clean and free from water, scale, dirt, oil residues etc. to reveal corrosion, deformation, fractures, damages, or other structural deterioration as well as the condition of the coating. However, those areas of structure whose renewal has already been decided by the owner need only be cleaned and descaled to the extent necessary to determine the limits of the areas to be renewed.

4.4 Sufficient illumination shall be provided to reveal corrosion, deformation, fractures, damages or other structural deterioration as well as the condition of the coating.

4.5 Where soft or semi-hard coatings have been applied, safe access shall be provided for the surveyor to verify the effectiveness of the coating and to carry out an assessment of the conditions of internal structures which may include spot removal of the coating. When safe access cannot be provided, the soft or semi-hard coating shall be removed.

4.6 The surveyor(s) shall always be accompanied by at least one responsible person, assigned by the owner, experienced in tank and enclosed space inspection.

# 5. Provisions and method of access to structures

This section of the survey programme shall indicate any changes relating to (and update) the information on the provisions and methods of access to structures provided in the survey planning questionnaire.

| Tank No. | Structure | C(Cargo)/ B(Ballast) | Permanent Means of Access | Temporary staging | Rafts | Ladders | Direct access | Other means (please specify) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F.P. | Fore Peak |  |  |  |  |  |  |  |
| A.P. | Aft Peak |  |  |  |  |  |  |  |
| Wing Tanks | Underdeck |  |  |  |  |  |  |  |
|  | Side shell |  |  |  |  |  |  |  |
|  | Bottom transverse |  |  |  |  |  |  |  |
|  | Longitudinal |  |  |  |  |  |  |  |
|  | Transverse |  |  |  |  |  |  |  |
| Centre Tanks | Underdeck |  |  |  |  |  |  |  |
|  | Bottom transverse |  |  |  |  |  |  |  |
|  | Transverse |  |  |  |  |  |  |  |

Note: To be filled with “X” in all columns except for “Other means (please specify)” as applicable.

5.1 For overall surveys, means shall be provided to enable the surveyor to examine the hull structure in a safe and practical way.

5.2 For close-up surveys, one or more of the following means for access, acceptable to the Surveyor, shall be provided:

(a) Permanent staging and passages through structures;

(b) Temporary staging and passages through structures;

(c) Hydraulic arm vehicles such as conventional cherry pickers, lifts and moveable platforms;

(d) Boats or rafts;

(e) Portable ladders; and/or

(f) Other equivalent means.

5.3 Surveys of tanks by means of boats or rafts shall only be undertaken with the agreement of the surveyor, who shall take into account the safety arrangements provided, including weather forecasting and ship response in reasonable sea conditions and provided the expected rise of water within the tank does not exceed 0.25 m.

5.4 When rafts or boats will be used for close-up survey the following conditions shall be observed:

(1) Only rough duty, inflatable rafts or boats, having satisfactory residual buoyancy and stability even if one chamber is ruptured, shall be used;

(2) The boat or raft shall be tethered to the access ladder and an additional person shall be stationed down the access ladder with a clear view of the boat or raft;

(3) Appropriate lifejackets shall be available for all participants;

(4) The surface of water in the tank shall be calm (under all foreseeable conditions the expected rise of water within the tank shall not exceed 0.25 m) and the water level either stationary or falling. On no account shall the level of the water be rising while the boat or raft is in use;

(5) The tank or space must contain clean ballast water only. Even a thin sheen of oil on the water is not acceptable;

(6) At no time shall the water level be allowed to be within 1 m of the deepest under deck web face flat so that the survey team is not isolated from a direct escape route to the tank hatch. Filling to levels above the deck transverses shall only be contemplated if a deck access manhole is fitted and open in the bay being examined, so that an escape route for the survey party is available at all times. Other effective means of escape to the deck may be considered;

(7) If the tanks (or spaces) are connected by a common venting system, or Inert Gas system, the tank in which the boat or raft is to be used is to be isolated to prevent a transfer of gas from other tanks (or spaces).

5.5 Rafts or boats alone may be allowed for inspection of the under deck areas for tanks or spaces if the depth of the webs is 1.5 m or less.

5.6 If the depth of the webs is more than 1.5 m, rafts or boats alone shall be allowed only:

(1) When the coating of the under deck structure is in GOOD condition and there is no evidence of wastage; or

(2) If a permanent means of access is provided in each bay to allow safe entry and exit. This means :

i. Access shall be direct from the deck via a vertical ladder and a small platform fitted approximately 2 m below the deck in each bay; or.

ii. Access to deck from a longitudinal permanent platform having ladders to deck in each end of the tank. The platform shall, for the full length of the tank, be arranged level with, or above, the maximum water level needed for rafting of under deck structure. For this purpose, the ullage corresponding to the maximum water level shall be assumed not more than 3m from the deck plate measured at the midspan of deck transverses and in the middle length of the tank.

If neither of the above conditions are met, then staging or other equivalent means shall be provided for the survey of the under deck areas.

5.7 The use of rafts or boats alone in 5.5 and 5.6 does not preclude the use of boats or rafts to move about within a tank during a survey.

# 6. List of equipment for survey

The equipment that will be made available for carrying out the survey and the required thickness measurements identify and list below:

(1) Thickness measurements shall normally be carried out by means of ultrasonic test equipment. The accuracy of the equipment shall be proven to the surveyor as required.

(2) One or more of the following fracture detection procedures may be required if deemed necessary by the Surveyor:

- Radiographic equipment

- Ultrasonic equipment

- Magnetic particle equipment

- Dye penetrant

(3) Explosimeter, oxygen-meter, breathing apparatus, lifelines, riding belts with rope and hook and whistles together with instructions and guidance on their use shall be made available during the survey. A safety checklist shall be provided.

(4) Adequate and safe lighting shall be provided for the safe and efficient conduct of the survey.

(5) Adequate protective clothing shall be made available and used (e.g. safety helmet, gloves, safety shoes, etc.) during the survey.

# 7. Survey requirements

7.1 Overall survey

This section of the survey programme shall identify and list the spaces that shall undergo an overall survey for this ship in accordance with CCS Rules and 2011 ESP Code.

(1) Cargo Tanks:

(2) Ballast Tanks:

(3) Fuel Oil Tanks:

(4) Lube Oil Tanks:

(5) Fresh Water Tanks:

(6) Other Tanks/ Spaces:

7.2 Close-up survey

This section of the survey programme shall identify and list the hull structures that shall undergo a close-up survey for this ship in accordance with CCS Rules and 2011 ESP Code.

# 8. Identifications of tanks for tank testing

This section of the survey programme shall identify and list the cargo holds and tanks that shall undergo tank testing for this ship in accordance with CCS Rules and 2011 ESP Code.

| Cargo Tanks |  |
| --- | --- |
| Ballast Tanks: |  |
| Fuel Oil Tanks: |  |
| Lube Oil Tanks: |  |
| Fresh Water Tanks: |  |
| Others: (if any) |  |

Notes:

(1) Tank testing of fuel oil, lube oil and fresh water tanks may be specially considered based on a satisfactory external examination of the tank boundaries, and a confirmation from the Master stating that the pressure testing has been carried out according to the requirements with satisfactory results.

(2) The testing of double bottom tanks and other spaces not designed for the carriage of liquid may be omitted, provided a satisfactory internal examination together with an examination of the tank top is carried out.

(3) Boundaries of ballast tanks are to be tested with a head of liquid to the top of air pipes.

(4) Boundaries of cargo tanks are to be tested the highest point that liquid will rise under service conditions.

(5) Boundaries of fuel oil, lube oil and fresh water tanks are to be tested with a head of liquid to the highest point that liquid will rise under service conditions.

(6) Cargo tank testing carried out by the vessel’s crew under the direction of the Master may be accepted by the surveyor provided the following conditions are complied with:

a) A tank testing procedure has been submitted by the owner and reviewed by the Society prior to the testing being carried out;

b) The tank testing is carried out prior to the overall survey or close-up survey;

c) The tank testing has been satisfactorily carried out within special survey window not more than 3 months prior to the date of the survey on which the overall or close up survey is completed;

d) The tank testing has been satisfactorily carried out and there is no record of leakage, distortion or substantial corrosion that would affect the structural integrity of the tank;

e) The satisfactory results of the testing is recorded in the vessel’s logbook.

f) The internal and external condition of the tanks and associated structure are found satisfactory by the surveyor at the time of the overall and close-up survey.

# 9. Identification of areas and sections for thickness measurements

This section of the survey programme shall identify and list the areas and sections where thickness measurements shall be taken in accordance with CCS Rules and 2011 ESP Code.

# 10. Minimum thickness of hull structures

This section of the survey programme shall specify the minimum thickness for hull structures of this ship that are subject to survey, (indicate either (a) or preferably (b) if such information is available):

(a)		For non-CSR Tankers, determined from the Appendix 3.1 Corrosion And Wastage Allowance of Hull Structure and the original thickness according on the hull structure plans of the ship;

(b)		For non-CSR Tankers, given in the table(s) listed in Appendix 3.2 Minimum Thickness Table of Hull Structure.

(c) 		For vessels built under IACS Common Structural Rules, the renewal thickness of the hull structure elements shall be indicated in the appropriate drawings.

# 11. Thickness measurements firm

This section of the survey programme shall identify changes, if any, relating to the information on the thickness measurement firm provided in the survey planning questionnaire.

Name of firm:

Address:

# 12. Damage experience related to the ship

This section of the survey programme shall provide details of the hull damages for at least the last three years in way of the cargo holds, ballast tanks and void spaces within the cargo length area, using the tables provided below. These damages shall be subject to survey.

Hull damages sorted by location for this ship

| Tank or space number or area | Possible cause, if known | Description of the damages | Location | Repair | Date of repair |
| --- | --- | --- | --- | --- | --- |

Hull damages for sister or similar ships (if available) in the case of design related damage

| Tank or space number or area | Possible cause, if known | Description of the damages | Location | Repair | Date of repair |
| --- | --- | --- | --- | --- | --- |

# 13. Areas identified with substantial corrosion from previous surveys

This section of the survey programme shall identify and list the areas of substantial corrosion from previous surveys.

# 14. Critical structural areas and suspect areas

This section of the survey programme shall identify and list the critical structural areas and the suspect areas, when such information is available.

The approved Structure Access Manual   □No.

Suspect areas□

- Applicable and in order     Not applicable

| List of Critical Areas for Tanks |  |
| --- | --- |
| Cargo tanks | connections of primary structures, such as inner bottom, long. bulkhead, vertical web, cross ties, stringers, web frames, deck transvers, particularly end bracket toes |
| Cargo tanks | boundaries of corrugations and bulkhead stools particularly in way of shelf plates, deck, inner bottom, etc |
| Cargo tanks/ Hopper tanks | knuckle connection between inner bottom plating and bilge hopper sloping plating，particularly connected hopper tank web frames, double bottom side girders and floors |
| Cargo tanks/ Hopper tanks | knuckle connection of bilge hopper sloping plating to inner hull longitudinal bulkhead, particularly connected web plating and horizontal girders |
| Wing tanks/ Hopper tanks/ Double-bottom tanks | connections of longitudinals to transverse web frames, floors and transverse bulkheads |
| Main deck | ends of deck transverse, if deck structures fitted on top of deck |

# 15. Other relevant comments and information

This section of the survey programme shall provide any other comments and information relevant to the survey.

# Appendices

Appendix 1 - List of Plans

CCS Rules require that main structural plans of cargo and ballast tanks (scantling drawings), including information on regarding use of high tensile steel (HTS), clad steel and stainless steel to be available. This appendix of the survey programme shall identify and list the main structural plans which form part of the survey programme.

Appendix 2 - Survey Planning Questionnaire

The Survey Planning Questionnaire, which has been submitted by the owner, shall be appended to the survey programme.

Appendix 3 - Other documentation

This part of the survey programme shall identify and list any other documentation that forms part of the survey programme.

.1	Corrosion And Wastage Allowance of Hull Structure , as referred to Paragraph 10 (a) “Minimum thickness of hull structures” is attached to this survey programme. ------------------------------------------------------------------------------

.2	Minimum Thickness Table of Hull Structure, as referred to Paragraph 10 (b) “Minimum thickness of hull structures” is attached to this survey programme. -----------------------------------------------------------------------------------------

The present survey programme is prepared by the owner in co-operation with China Classification Society.

Date: ( ) (name and signature of authorized owner's representative)

Date: ( ) (name and signature of Surveyor to CHINA CLASSIFICATION SOCIETY)

# Appendix 1 - List of Plans

1. Main structural plans (scantlings drawings) of cargo and ballast tanks, including information regarding use of high tensile steels (HTS), clad steel and stainless steel (for CSR ships these plans are to include for each structural element both the as-built and renewal thickness. Any thickness for voluntary addition is also to be clearly indicated on the plans. The midship section plan to be supplied on board the ship is to include the minimum allowable hull girder sectional properties for tank transverse section in all cargo holds)

| No. | Description |
| --- | --- |
| 1 | Midship Section and Typical Trans. BHD |
| 2 | Construction Profile & Decks |
| 3 | Shell Expansion |
| 4 | Transverse Bulkheads |
| 5 | Stem Construction |
| 6 | Stern Construction |
| 7 | Any other plans requested by the attending Surveyor |

2. Any other documentation that forms part of the plan

| No. | Name of documentation |
| --- | --- |
| 1 | General Arrangement |
| 2 | Capacity Plan |

中  国  船  级  社

CHINA CLASSIFICATION SOCIETY

SURVEY PLANNING QUESTIONNAIRE

FOR TANKERS

# Appendix 2 - Survey Planning Questionnaire to Survey Programme

The following information will enable the owner in co-operation with China Classification Society to develop a survey programme complying with the requirements of CCS Rules and 2011 ESP Code. It is essential that the owner provides, when completing the present questionnaire, up-to-date information. The present questionnaire, when completed, shall provide all information and material required by CCS Rules and 2011 ESP Code.

1. General Particulars

| Ship's name: |  |
| --- | --- |
| IMO number: |  |
| Flag State: |  |
| Port of registry: |  |
| Owner: |  |
| Recognized organization: | China Classification Society |
| Gross tonnage: |  |
| Deadweight (metric tonnes): |  |
| Date of build: |  |

2. Information on access provision for close-up surveys and thickness measurement

The owner shall indicate, in the table below, the means of access to the structures subject to close-up survey and thickness measurement. A close-up survey is an examination where the details of structural components are within the close visual inspection range of the attending surveyor, i.e. preferably within reach of hand.

| Tank No. | Structure | C(Cargo)/ B(Ballast) | Permanent Means of Access | Temporary staging | Rafts | Ladders | Direct access | Other means (please specify) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F.P. | Fore Peak |  |  |  |  |  |  |  |
| A.P. | Aft Peak |  |  |  |  |  |  |  |
| Wing Tanks | Underdeck |  |  |  |  |  |  |  |
|  | Side shell |  |  |  |  |  |  |  |
|  | Bottom transverse |  |  |  |  |  |  |  |
|  | Longitudinal |  |  |  |  |  |  |  |
|  | Transverse |  |  |  |  |  |  |  |
| Centre Tanks | Underdeck |  |  |  |  |  |  |  |
|  | Bottom transverse |  |  |  |  |  |  |  |
|  | Transverse |  |  |  |  |  |  |  |

Note: For all columns except the column mark with “Other means (please specify)”, “X” is to be filled in as applicable.

Details of the inert gas plant and tank cleaning procedures:

(a)Inert Gas System installed:             Yes      No

-Details of inert gas plant:

-Indicate average oxygen content during inerting:

(b)Cargo Tank Cleaning Procedures:

i. Indicate the frequency of the tank washing, especially uncoated tanks:

- After cargo discharging:

-Before dry docking:

ii. Washing medium used:

-Crude Oil:                         Yes      No

-Heated seawater:                    Yes      No

-Other medium (specify):

For oil tankers: History of cargo with H2S content or heated cargo for the last 3 years together with indication as to whether cargo was heated and where available, Marine Safety Data Sheets (MSDS)*： For chemical tankers: History of heated cargo for the last 3 years together with indication as to whether cargo was heated.

* Refer to resolution MSC.150(77) on Recommendation for material safety data sheets for MARPOL Annex I cargoes and marine fuel oils.

3. Owner’s inspection

Using a format similar to that of the table below (which is given as an example), the owner shall provide details of the results of their inspections, for the last 3 years on all cargo holds and ballast tanks and void spaces within the cargo area, including peak tanks.

| Tank No. | Corrosion Protection ① |  |  |  |  | Coating extent ② |  |  |  | Coating Condition ③ |  |  |  | Structural Deterioration ④ | Tank Damage History ⑤ |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | HC | SH | SC | A | NP | U | M | L | C | G | F | P | RC | Y/N | DR | L | CV |
| Cargo centre tanks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cargo wing tanks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Slop |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ballast tanks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aft peak |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fore peak |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Miscellaneous spaces |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Abbreviation:

① HC-N: Hard coating not subject to PSPC and PSPC (Void Space)

HC-B: Hard coating applied in dedicated seawater ballast tanks (PSPC)

HC-C: Hard coating applied in cargo oil tank spaces (PSPC)

HC-V: Hard coating applied in void spaces (PSPC)

SH = Semi Hard Coating    SC = Soft coating    A = Anodes    NP = No protection

② U=Upper part    M=Middle part    L=Lower part    C=Complete

③ G=Good    F=Fair    P=Poor    RC=Recoated (during the last 3 years)

④ Y= Findings recorded, description of findings shall be attached to this questionnaire.     N = No findings recorded

⑤ DR=Damage & Repair    L=Leakage    CV= Conversion, description shall be attached to this questionnaire.

Notes:

(1) To be filled with applicable type of hard coating in column mark with “HC”;

(2) To be filled with “Y” or “N” in column mark with “Structural Deterioration ④”;

(3) To be filled with “X” in other columns as applicable.

Name of owner’s representative:

Signature:

Date:

Name of owner’s representative:

Signature:

Date:

4. Reports of Port State Control inspections

| List the reports of port State control inspections containing hull structural related deficiencies and relevant information on rectification of the deficiencies: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date of inspection | Port of inspection | Ship detained (Yes / No) | Description of hull structural related deficiencies / PSC report No. | Description of deficiencies rectification / PSC report No. |

5. Safety Management System

| List nonconformities related to hull maintenance, including the associated corrective actions: |  |  |  |
| --- | --- | --- | --- |
| Description of hull structural related non-conformities | Non-conformities given by | Corrective actions | Date of verification |

6. Name and address of the approved thickness measurement firm

| Name of firm: |  |
| --- | --- |
| Address: |  |

# Appendix 3.1 - Corrosion And Wastage Allowance of Hull Structure

This Appendix does not apply to the ships constructed in accordance with PART NINE of CCS Rules (2012 version) and its 2013 and 2014 amendments or PART NINE of CCS Rules (2015 version) and its subsequent versions (including amendments).

1 For a ship constructed in accordance with CCS rules and the keel of which was laid on or after 15 January 1983, the renewal thickness of hull plating and structural members is not to be less than the value obtained by multiplying their as-built thickness and the relevant percentage shown in Table 1 below:

Table 1

| Structural member | Minimum renewal thickness |  |
| --- | --- | --- |
|  | L≥90 m | L<90 m |
| ① Strength deck plating, side shell, top strake, bilge strake, bottom shell, flat plate keel, inner bottom, continuous longitudinal bulkhead, hopper tank and topside tank plating. ② Main longitudinal continuous members, e.g. deck girders, hatch side girders, side girders, bottom girders, bulkhead girders, continuous hatch coamings. ③ Main transverse members, e.g. side frame webs, deck transverses, double plate floors, bulkhead webs, watertight and oiltight transverse bracket plates. ④ Transverse bulkhead plating in holds, upper and lower bulkhead stool sloping plating, watertight bulkhead plating in deep tanks. | 80% | 75% |
| Other plating and members, e.g. deck within line of openings, deck longitudinals, side longitudinals, bottom longitudinals, inner bottom longitudinals, bulkhead longitudinals, face plates of frames, brackets of members, hatch covers, non-continuous hatch coamings, sea chests. | 75% | 70% |

2 For tanks constructed in accordance with CCS Rules and the scantlings of which as required by CCS Rules are indicated in their plans, the thickness reduction of hull plating and structural members caused by corrosion and wastage is not to be more than the value obtained by multiplying the thickness specified in CCS Rules and the relevant percentage shown in Table 1 above.

# Appendix 3.2 - Minimum Thickness Table of Hull Structure

Minimum thickness of hull structures is given in the following table:

| Area or location | Original as-built thickness (mm) | Minimum thickness (mm) | Substantial corrosion thickness (mm) | Remarks |
| --- | --- | --- | --- | --- |
| Deck |  |  |  |  |
| Plating |  |  |  |  |
| Longitudinals |  |  |  |  |
| Longitudinal girders |  |  |  |  |
| Bottom |  |  |  |  |
| Plating |  |  |  |  |
| Longitudinals |  |  |  |  |
| Longitudinal girders |  |  |  |  |
| Inner bottom |  |  |  |  |
| Plating |  |  |  |  |
| Longitudinals |  |  |  |  |
| Longitudinal girders |  |  |  |  |
| Ship side |  |  |  |  |
| Plating |  |  |  |  |
| Longitudinals |  |  |  |  |
| Longitudinal girders |  |  |  |  |
| Longitudinal bulkhead |  |  |  |  |
| Plating |  |  |  |  |
| Longitudinals |  |  |  |  |
| Longitudinal girders |  |  |  |  |
| Transverse bulkheads |  |  |  |  |
| Plating |  |  |  |  |
| Stiffeners |  |  |  |  |
| Transverse web frames, floors and stringers |  |  |  |  |
| Plating |  |  |  |  |
| Flanges |  |  |  |  |
| Stiffeners |  |  |  |  |
| Cross ties |  |  |  |  |
| Flanges |  |  |  |  |
| Webs |  |  |  |  |

Remarks: