| - This checklist provides general guidance (not exhaustive) for review by the Management Company, Master, and Chief Engineer in preparation for PSC inspections. - Complete this checklist at least once every three (3) months prior to entering a US port, Australian port, or any port within the Paris MOU region. Report any items marked "No" to the CCS customer manager. - Pay special attention to all key points within any item marked in red. Any key point marked in red requires focus on it. - Remark: Proper Handling of Deficiencies: 1. If the result is "No," please formulate corrective and preventive measures according to the company's non-conformance handling procedures, and specify the time frame for rectification to prevent any delays. 2. When a Port State Control Officer (PSCO) attends the vessel, inform him/her of any defects identified, along with the corrective actions taken and measures implemented to prevent recurrence. |  |  |  |
| --- | --- | --- | --- |
| No. | Item | Key point | Result |
|  | Embarking |  |  |
|  | Hull structure/LL Marks | - Free of apparent corrosion, pitting, cracking, deformation, fracture; - Clearly marked and correspond with ILLC; carriage of cargoes without overload |  |
|  | Gangway/Accommodation ladder/ Vessel access/ Pilot Ladder | - Without any defects (fractured steps, side ropes etc.) /Protection net - Guard all time /visitor check/visitor log - Security level presentation / restricted area marked clearly and locked - Max. Load/person& angle mark - Lifebuoy with light and lifeline in the vicinity of accommodation ladder |  |
|  | Master’s office/Deck office |  |  |
|  | Certificates | - E-/hard copies available/valid/endorsed - Servicing certificates for firefighting and lifesaving equipment - Seafarers certificates: COC/endorsement/STCW requirements for specific ship type/MLC required |  |
|  | Survey reports | - Survey status available, No overdue COC/SMG/CMG |  |
|  | PSC/FSC inspection records | - All deficiencies rectified effectively/No reoccurrence |  |
|  | MLC documentation | - Minimum age: no seafarer under 16; no night work under 18 - Rest hours records: Arrival/departure/bunkering/on watch - Month payment slip: pay monthly & full with signature - On-board complaint procedures: in working language/no complaint - SEA: only one set of SEA valid for each seafarer/CBA (if applicable) - Manning agencies: licensed |  |
|  | Manuals（check the applicable items） | - Stability booklet: on board - Damage control plan: on board - Loading Sequence Manual: on board - Cargo securing manual: on board - grain loading manual: on board - Emergency towing manual - Survey report file(ESP)：ESP plan/Reports/TM report - SOPEP/SMPEP: Updated contact list/approval by flag or RO - SEEMP I&II&III，MRV EU&FUELEU MP(if any): on board - NOx technical file: parameters records/ EIAPP certificates - Coating technical file - required for PSPC: on board - Oil record /Garbage record /Ballast water record book: Accurate entries accordingly - Ballast water management plan/Garbage management plan: consistency with the fact - EGCS technical files：Records - P&A Manual/Cargo Operation Manual: on board |  |
|  | Cargo information | - Cargo character/MSDS/loading plan/loading records/cargo weight evidences - Protection measures and training records (if applicable) |  |
|  | Bridge |  |  |
|  | Nautical/ITU publication (Lights list/Tide table/Sail. Dir./IAMSAR Manual etc.) | - IMO publications and flag Administration regulations - E-publication with E/G power; - ITU publication: latest and in place |  |
|  | Nautical charts/ECDIS | - Next Voyage charts: Available and updated - ECDIS: Operation test/Maintain periodical check/Operation under both main and emergency source of power; Familiarization/the training records of Deck Officers /Safety setting/UKC setting - Coastal warnings (Navtex) updated |  |
|  | Log book/Training record | - Drill details accordance with training records files : Abandon ship/Fire drill 1Mon/Lifeboat launching 3Mon/ Free-fall lifeboat drill Simulated launching -6Mon/ Rescue boat 1Mon (at least 3Mon with reason) /Emergency steering drill 3Mon/Davit-launched liferaft 4Mon/Entering to enclosed spaces-2Mon /oil spill-3Mon - 2-month interval SOLAS lecture records - On watch records/Master on call/BNWAS records |  |
|  | Watch keeping schedules/Muster list/pre-arrival information | - Watch keeping schedules/muster list posted - Pre-arrival information in place. |  |
|  | Radio equipment, GMDSS, AIS, VHF/DSC, MF/HF, EPIRB, AIS, VDR, Radar Transponder, Echo sounder, Speed log, NAVTEX, MMSI number, LRIT, BNWAS, Radar, EGC | - Working in order: Fault alarm checking - MF/HF radio: test in Emergency/Reserve of power(BQ/RQ)/Familiarization/testing to the shore station - EPIRB: Test report/valid date of HRU/Location/Batteries - NAVTEX: Area/signal - LRIT: conformance test on board(Once a year required by Liberia & Panama) - SSAS：Function test - VHF: Battery check - BNWAS: Records/Function test - Radar: No signal loss/PM test - VDR: Batteries/Fault alarm - EGC: coastal area |  |
|  | Immersion suit/Lifejacket | - Condition (Glue/Zipper/Sewing parts, detached or sticked, etc) - Check type & size(Check match with approval certificate) - Immersion suit : unpacked and donned without assistance within 2 min |  |
|  | Standard magnetic/ GYRO compass | - No bubble/Deviation |  |
|  | Daylight signaling lamp | - Operation by emergency power at both sides and battery - Portable battery and at least 3 spare illuminants(2002.07.01) |  |
|  | Fire alarm & fire detection system | - Sampling test by emergency power, Extension alarm in 2 min. - No error or fault message displayed /disable |  |
|  | Navigation light panel/ Emergency lighting | - Alarm test by emergency power - Functioning - Pilot lamp |  |
|  | Main fire pump/Emergency fire pump starting panel in bridge (if applicable) | - Condition/Test |  |
|  | Water tight doors alarm (if applicable) | - Functioning Test/indicator |  |
|  | MOB lifebuoys | - Quick release/Self-igniting light/Expiration date/Weight |  |
|  | Navigational light | - Matt black paint/Required angles/Free of damage - NUC light functioning - Main power/emergency power |  |
|  | Sound signal | - Test/Air pipe condition |  |
|  | Bridge Steering gear Alarm Panel | - Power failure(Main & Emergency)/Control System Power Failure/Overload |  |
|  | Outside of Accommodation |  |  |
|  | Battery room | - Reserve condition for GMDSS/Recharging/Connecting cable/ protection kids/ expire dates |  |
|  | E/R funnel dampers & ventilators | - Funnel damper’s closing condition(witness aside) : local & remote control/No apparent light observed - Ventilator damper’s closing condition/indicators (if suspicion, open the observing hole to check it) - No leakage of control air pipe/operation wheel/seals - Free of corrosion |  |
|  | E/G room | - Heating equipment/Panels/Gauges/Functioning/Insulation of engine/Isolate valve from Fuel/Stand-by position - E/G ventilator: In good condition/ Free of excessive corrosion/ Control position/ Condition of interlock ventilation - Secondary start functioning test - Blackout test with test mode and/or interlock switch in E/R (running over 5 Min.) - Recover to stand-by position. - Fire extinguisher (Power >375Kw fixed fire-fighting sys.) |  |
|  | Inflatable liferaft | - Quantities/valid HRU/ Weak link connection/ no obstacle - Bindings cut or not/ Retro-reflective tape - Annual Report - Embarkation ladder |  |
|  | Davit (life boat, rescue boat & Davit-launched liferaft) | - No Excessive corrosion - Single arm davit for rescue boat/liferaft: two(2) means power in good working condition/Accumulator pressure check/Davit arm rotating angle/Limit switch - Davit for lifeboat: FPD check/lower lifeboat test - Davit for free fall lifeboat: Test without lifeboat/Davit lashing devices - Wire/sheave condition - Lifting Hook Assembly - Winch, motor & foundation: Bolt/nuts/Fall renew(5 year)/Winch Brake/working condition |  |
|  | lifeboat | - No Hole&Crack/Marking/Painting/Retro-reflective tape/ Window transparency/secure devise/drain valve in lowest point - Fire extinguisher/ Safety belt color/ EPIRB/ compass/ lighting/ water spray（if applicable）/Breathing air system(if applicable) - Inventory - Lifeboat Engine: Battery/ F.O level/starting &3Min running test/propulsion FWD/AFT - Rudder: full movement/manual movement - On-load release gear: properly reset properly reset with clear mark and release cable visually inspected without any strand breakage |  |
|  | Rescue boat(other than lifeboat) | - No Hole&Crack/Marking/Painting/Retro-reflective tape/ securing - Engine stop key in place - F.O. level & Tk/Battery recharged(if required) - Two independent engine starting system - Cooling water spray (in case of outboard engine) - On-load release gear: properly reset |  |
|  | Lifeboat lowering | - Drill Test /Remote lowering /Brake/ Limit switch test /Water-spray (if req.) |  |
|  | Emergency light and searchlight | - Function test & good condition/Not seized |  |
|  | Accommodation |  |  |
|  | Restricted area | - Marked clearly/locked |  |
|  | Fire doors | - The self-closing device/No holding hook/sign mark - Manual/automatic release function(if applicable) |  |
|  | Fire extinguishers | - Condition/service marked/position as fire control plan/spare |  |
|  | Fire detector/Manual call point (MCP) | - Sampling test with Audible/Visible alarm |  |
|  | Firefighter's outfits | - Separate stowage/condition/SCBA& pressurized spare/wearing Familiarization (put on) - Two two-way portable radiotelephone (explosion-proof type or intrinsically safe) - Annual report of Firefighter's outfits - Annual and 5 year report of SCBA |  |
|  | Fire detection system | - No alarm/clear monitor/detectors |  |
|  | Fixed fire-fighting systems (CO2) | - Bottle contents/test & check date/Flexible connecting hose/Safety pin in correct position/Remote Control Line/Pressure release valve and line/ Room key/Operation instruction/Means of interlocking/Weighting device - Pressure check of the control cylinders. - Crew familiarity (any person) - Cargo hold fixed smoke detection and/or extraction system (if applicable) in good order/functioning test |  |
|  | Fire Isolating valve for E/R and deck | - Location/open-close indicator/marking/close properly |  |
|  | Deck office & CCR(if applicable) | - Loading computer/BWMS & ballast operation panel - Cargo operation and control system etc. good condition - ODME/Fixed detection systems good condition - Gas detector: calibration Certificate/function test |  |
|  | Drawings | - Approved Fire Control Plan and Damage Control Plan posted - Enclosure for Fire Control Plan or booklet should be weathertight |  |
|  | Galley | - Hygienic/grease traps clean/ Temperature of refrigerator and freezer - Provision condition/refrigerated store rooms alarm - Fire damper/ventilator operated from outside - Fire extinguisher/fixed fire system/Fire blanket |  |
|  | MLC-related matters | - Heating, lighting, ventilation/ Sanitary facilities/Complaint procedure/wage slip - Hospital in good condition/oxygen bottle/medicine/medicine record - Adequate food/drinking water |  |
|  | Main Deck & Cargo Hold& Poop Deck |  |  |
|  | Shore connection | - Correct connection/Spare(fire-fighting) - Flange size and nuts(oily-water(six nuts)/sewage(four nuts) |  |
|  | Ventilator/air pipe | - closing appliances/dampers not seized/gaskets in order/floats in the heads not seized/Wire gauzes fitted to fuel oil tanks air pipes - Ventilator heads without seriously rusted/wasted and missing bolts. |  |
|  | Fire main line/Foam line/IGS line/Fixed CO2 gas pipeline/ Foam valves(if applicable) | - Corrosion/No leakage/Valve opeation/No Patching |  |
|  | Pilot transfer arrangements | - Condition/no fractured steps - Permanent marking on pilot ladder - Pilot ladders: manropes for correct size/No defect/No excessive clearance between chocks and steps, spreader bent. |  |
|  | Main deck | - Corrosion/pitting/cracking/deformation/fracture/painting |  |
|  | Small hatch | - Corrosion/ Closing device/ Hole /Weathertight |  |
|  | Hatch cover | - Corrosion/Weathertight/Working condition/the rubber seals without exceeded groove - No leakage from hydraulic lines/motors - Secure devices |  |
|  | Bulwark and handrail | - Corrosion/Crack/deformation - On-deck safety access to bow |  |
|  | Cargo holds & coaming | - Free of apparent corrosion/ pitting/ cracking/ deformation/ fracture/ painting - access ladder: No corrosion/ deformation/ fracture - Ingress water detect test(if applicable) |  |
|  | Man hole cover | - Condition/bolt/bolt hole |  |
|  | Windlass/Winch/Mooring line | - Corrosion/ Brake pad/Drum condition - No leakage from hydraulic line - Mooring line/rope: condition/ maintenance record |  |
|  | Operating instructions | - Operating instructions for equipment (life boat, rescue boat, liferaft, etc.) posted and near emergency light - Operating instructions for fixed fire-fighting system posted and accurate |  |
|  | Paint store | - Fire extinguisher: In place/type - Fixed spray system (if applicable): Nozzle check/Test - Fire detector: Condition/test - Explosion-proof light - Ventilator: |  |
|  | Bow life-raft | - Installation position/ emergency lighting and boarding facilities |  |
|  | Bosun Store |  |  |
|  | Hawse pipe | - Corrosion/No leakage |  |
|  | Chain locker wall | - Corrosion/Watertight |  |
|  | Anchor chain abandoner | - Corrosion - Operation manual posting - The pin is active and easy to loosen |  |
|  | Collision bulkhead isolation valve | - Operation manual posting/Operation test/Crew familiarization |  |
|  | Weather tight door | - Corrosion/Packing/Closing device - No penetrating light into |  |
|  | Lifejacket | - Quantity/Lighting/Ship name/Port of registry/Condition |  |
|  | Immersion suit | - Condition (Glue/Zipper/Sewing parts, detached or sticked, etc) - Check type & size(Check match with approval certificate) |  |
|  | Steering Gear Room & Emergency Fire Pump Room |  |  |
|  | Fire doors/air-locked doors | - No holding back /condition/packing/self-closing device |  |
|  | Insulation | - No Missing/damage |  |
|  | Emergency lighting | - Test |  |
|  | Steering gear | - Hand rails and non-slip surface - No Oil leakage/L.O tank & alarm for hyd. Pump/Emergency steering - Steering gear room communication with the bridge - Test in all mode - Angle readings as the bridge’s and heading information - Operation instruction in place |  |
|  | Emergency fire pump | - Protection of suction or discharge piping penetrating E/R - Pipe protection(Pipes in E/R)/Valves remote operation - Starting/vacuum pump condition/Suction pressure/Two jets of water (from Forward/Bridge)/checking Isolation valve/pressure gauge |  |
|  | Water mist fire-fighting system (may located in E/R) | - Water tank lowest level mark and alarm - Auto-mode position - Manually release operation instruction and tools in place - Auto-mode test at top of G/E or/and boiler(when check boiler &G/E) - Audible/Visible alarm indication |  |
|  | Engine Room |  |  |
|  | Engine Room Doors | - Emergency exits clear and free from obstructions. - Fire doors properly closed with no holdbacks. |  |
|  | Engine Room | - Clean and tidy; - Movable parts protected/guarded; - No flying electrical cables or junction boxes; - Danger of electrical shocks clear out; - Personal protection warnings and protective equipment (helmets, ear-plugs, goggles, gloves, etc.) - Piping: No patches/Temporary repair with class approval - A-60 fire insolation intact - Fuel oil sampling point(s) (2025.01.01) |  |
|  | Engine Control Room | - Main switchboard condition - A/E parallel test - A/E control position indicator - Emergency generator position indicator - Interlock switch between E/G & Main Switchboard - Fire detector - Obstacle behind the main switchboard - Alarm monitoring system(any alarm was dealt with correctly) - Alarm monitor |  |
|  | Main Engine & Auxiliary Engines | - Clean and free from leakage; - Hot surfaces insulated; - Safety valves; - Temp. & press. Gauges; - Fuel leakage alarm; - M/E Oil mist detector; - Remote and emergency starting system - Quick close valves for cutting off fuel supply - Jacketed pipe for high pressure fuel(no break, drainage alarm) - EPL/shapoli: position/records - EGC/DCR/EGR(if applicable) - NOx Tier III/ SOx emission control zone: records/control |  |
|  | Boilers | - valves, water, steam and fuel systems, gauge glass mountings; - boiler and piping insulated; - local and remote control of safety devices - auto and manual control system (AUTO-0 keeps auto mode) - visual alarms/indicators - Steam pipe insulation |  |
|  | Electrical Safety | - check feeder panel insulation in engine control room - check feeder panel for defective insulation meter - check portable electrical equipment (lights, extension cords, cutting saw etc) in unsafe condition (protection cover and earth connection missing, exposed wire) - check for missing insulated matting behind switchboard |  |
|  | Fire detector | - Detector test - Test kits in place - Audible/Visible alarm |  |
|  | Portable fire extinguisher | - Contents replacing date - Working test(Sampling) - Hydro test date/Spares in order | 选择一项。 |
|  | EEBD | - Instruction - Storage condition - Proper Air Pressure |  |
|  | Emergency escape trunk | - Self-closing fire door (Check fire integrity, repair hole on door) - Insulation condition (Extension of insulation at intersections & terminal points) - Emergency lighting - Internal dimension of enclosure, at least 800mm X 800mm (Keel laid on or after 2002.07.01) |  |
|  | Quick closing valve | - Functioning test (if possible) - Working condition - Any hold-back or blockage |  |
|  | Insulation, lagging | - No missing/No breakaway/omission - Hot space insulation - No oil soaked |  |
|  | Oil filtering equipment | - Operation test of oil filtering equipment (15ppm alarm, auto-stop & etc.) /operation familiarity/recycling valves - Sample valve and sample pipe arrangement - Use maker’s filter & spare filter - Close the discharge valve - Operation manual posting - Electrical record meeting with the hard record - The discharge overboard valve(locked) - No direct line for discharging oily bilge |  |
|  | E/Room lowest floor | - Bilge quantity/Bilge well alarm test - Oil/water leakage - No residual and oily rags - Sounding pipe and self-closing devices - M/E shaft & seals: No oil leakage |  |
|  | Ballast water management system | - Operation test - Check operation log & alarm history, Cross-check with the Ballast Water Management Record Book(at least english), Confirm no illegal bypass operations |  |
|  | Sewage equipment | - Hull corrosion - Level alarm - Operating and alarm test/ Crew’s familiarity for equipment - Air diffuser /circulation line/Chlorine tablet - Confirmation of calculation sheet on sewage discharge ratio - Untreated sewage direct overboard valve kept close in port. |  |
|  | Incinerator | - Alarms/Safety devices/interlock |  |
|  | Bulk carrier additional items |  |  |
|  | Cargo | - Shippers declaration on board - Loading instruments/Loading Booklet - Master’s Report - Monitoring equipment: Oxygen, Gas, pH and Temperature Operation test - Procedures and responsibilities for cargo operations |  |
|  | Water ingress system | - Test/Indicator - Pumping systems |  |
|  | Dewatering system | - Sensor test/Remote control Valve in order |  |
|  | Oil/Gas/Chemical tanker additional items |  |  |
|  | I.G. System in E/R | - Up take valves (for flue gas system): Discharge and recirculation valves in order and with remote control (pneumatic) in order: no leakage. - I.G. Fans (at least two): in working condition. - Scrubber water supply: by at least two pumps/sight glass in order and not blinded. - Fixed oxygen analyzer: provided with span gas for calibration. - I.G. generator (if fitted) in good order including built in burner, ventilator and scrubber. - Control instrument/including alarm sensor: fitted in working condition. |  |
|  | I.G. System on Deck | - I.G. line on main deck: in order/No corrosion/no holes/no unauthorized repair (soft patch, clamps). - Deck seal: in order with no leakage. - Mechanical automatic non return valve: in place after deck seal. - P/V Breaker filled in (glycol) and provided with level indicator. - Delivery line to cargo tank: fitted with valve provided with suitable means of locking. - Shore connection: provided with blank flange fully bolted and clearly marked. - Portable oxygen analyzers: available and in working condition/ calibration record. |  |
|  | Pumproom and pumproom ventilation system | - Ventilation interlock: in working condition - ODME system: test/records check |  |
|  | Cargo control room | - I.G. system control panel: Synoptic control panel in working condition: lamps, lamp test function, alarm buzzer. - I.G. content and I.G. pressure recorder: paper and ink available. - Test of system including check of alarm and safety devices: system trip – automatic switch to recirculation mode. - Check the proper setting of alarms/working condition of recorder/ proper working condition of automatic control of valves. |  |
|  | RO-RO |  |  |
|  | Shell Doors | - Locking and Sealing Arrangements |  |
|  | Vehicle Deck | - Ramp/Bulwarks/Guard Rails: No corrosion/No fracture - Cargo Securing: properly - Vehicle Deck drainage: test - Fire Protection Arrangements |  |
|  | Container ship |  |  |
|  | Cargo Securing Arrangements | - Lashing/ fittings: maintained properly/compliance with Manual - Cargo Securing: properly |  |
|  | Navigation bridge visibility | - The view on the bridge deck clear |  |
|  | Drill exercise |  |  |
|  | Fire Drill: - The fire founder's reporting to bridge and initial fire extinguishing in small fire only - Communication between the fire location and bridge - Alarm sounding correctly /Prompt muster at designated stations within 3 minutes/ roll call /Crew wearing appropriate (PPE) - Firefighting team dons fireman’s outfits and SCBA correctly within 5 minutes/SCBA checks: Air pressure, functioning gauges, donning procedure, communication test. - Effective communication: Radios working, standardized phrases used, no delays in reporting/Situation reports relayed promptly to the bridge (simulated fire location, type, status). - Correct selection/display of firefighting equipment (hoses, nozzles, extinguishers, foam, etc.) for the simulated fire type. - Demonstration of boundary cooling techniques. - Simulated activation of fixed firefighting system (CO₂, foam) with emphasis on safety procedures (evacuation, dampers, alarms). - Rescue team equipped and ready for immediate deployment. /Simulated first aid for an "injured" crew member./Emergency escape routes demonstrated/cited by crew. - Conclusion & Debrief: Orderly drill termination by Incident Commander. /Debriefing held with all crew to review actions, timing, and improvements./Equipment properly stowed and ready for reuse after drill. |  |  |
|  | Abandon Ship Drill: - Initiation & Alarm: Correct activation of the abandon ship signal /Crew must recognize the signal immediately and proceed to muster stations. - Muster & Accountability: Full crew assembly at assigned stations within 2–3 minutes /Roll call to confirm all personnel in place/ Crew wearing lifejackets correctly and carrying immersion suits (if required). - Preparation of Lifeboats (LB) / Rescue Boats (RB): Crew demonstrates rapid clearing of launching areas (remove lashings, safety pins, harbour locks)./Engine started within 2 minutes (for motor-propelled craft). Davit-launched craft: Winch brakes tested, falls rigged correctly. Free-fall craft: Safety mechanisms released, crew seated and secured. Life rafts (LR): Crew knows how to deploy/manually launch (if assigned). - Embarkation & Launching: Orderly embarkation/Lowering/launching/Simulated lowering of at least one lifeboat/Rescue boat launched/maneuvered (if feasible). Remark: On-load release gear visually checked before lowering. Crew familiar with disengaging hooks (hydrostatic/interlock functions tested). - Survival Equipment Checks: Lifeboat/Raft equipment: Crew confirms presence of: Pyrotechnics (rockets, flares). EPIRB /SART (tested if possible).Freshwater/rations, first aid kit./Thermal protective aids (TPAs)./Radio equipment: VHF tested on DSC Ch.70/16. - Command & Communications: Master maintains control from bridge. /Radio checks: Communication between lifeboat/rescue boat/bridge./"Mother ship" coordination: Signals (flags/lights) understood by crew. - Immersion Suit Proficiency: Donning time: Crew dons immersion suits within 2 minutes (PSCO may time randomly selected crew). - Conclusion & Debrief: Recovery: Safe retrieval of survival craft (winch operation, re-securing). Debrief: Master reviews drill performance, logs drill details (date, time, scenarios). Launch of the survival craft within required time period |  |  |
|  | - All other drills performed as per ISM SMS and lMO requirements, timely and correctly etc.. - Annual drill plan/log book/drills record. |  |  |